MPLS L3 VPN CONFIGURATION

Matiere: Conception et

Implémentation

d’un Réseau

Sécurisé

Code: NET379

Professeur: M. NOUGBEGNON ELISEE

Etudiant: AHLIDZA K. Emmanuel

Annee universitaire:2021-2022

IPNET INSTITUTE OF TECHNOLOGY

MPLS L3 VPN CONFIGURATION

In our scenario

- Service provider router P is the ISP core routers, run only MPLS

- Provider edge routers PE1 and PE2 connect to customer sites Venus and Gvtel

- Customer edge routers are at the edge of a customer site; they have no VPN

- awareness

- IGP like OSPF is running on P and PE routers to support LDP and BGP adjacencies

- within the provider network

- MP-BGP is running only PE routers

- eBGP is running between CE and upstream PE routers

Topologie réalisée

- Configuration

- P#sh run

- Building configuration...

- !

- ! Last configuration change at 00:07:42 UTC Wed Mar 16 2022

- !

- hostname P

- !

- interface Loopback0

- ip address 2.2.2.2 255.255.255.255

- !

- interface GigabitEthernet0/0

- ip address 103.13.148.2 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- interface GigabitEthernet0/1

- ip address 100.100.100.2 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- router ospf 1

- network 2.2.2.2 0.0.0.0 area 0

- network 100.100.100.2 0.0.0.0 area 0

- network 103.13.148.2 0.0.0.0 area 0

- !

- end

- P#

- PE1#sh run

- Building configuration...

- !

- ! Last configuration change at 02:16:46 UTC Wed Mar 16 2022

- !

- !

- hostname PE1

- !

- !

- ip vrf Emma

- rd 525:3

- route-target export 525:3

- route-target import 525:3

- !

- ip vrf Gvtel

- rd 525:2

- route-target export 525:2

- route-target import 525:2

- !

- ip vrf Venus

- rd 525:1

- route-target export 525:1

- route-target import 525:1

- !

- !

- interface Loopback0

- ip address 1.1.1.1 255.255.255.255

- !

- interface GigabitEthernet0/0

- ip address 103.13.148.1 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- interface GigabitEthernet0/1

- ip vrf forwarding Venus

- ip address 172.16.10.1 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- interface GigabitEthernet0/2

- ip vrf forwarding Gvtel

- ip address 172.16.20.1 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- interface GigabitEthernet0/3

- ip vrf forwarding Emma

- ip address 172.16.50.1 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- router ospf 1

- network 1.1.1.1 0.0.0.0 area 0

- network 103.13.148.1 0.0.0.0 area 0

- !

- router bgp 525

- bgp log-neighbor-changes

- neighbor 3.3.3.3 remote-as 525

- neighbor 3.3.3.3 update-source Loopback0

- neighbor 172.16.10.4 remote-as 100

- neighbor 172.16.20.5 remote-as 200

- neighbor 172.16.50.8 remote-as 500

- !

- address-family vpnv4

- neighbor 3.3.3.3 activate

- neighbor 3.3.3.3 send-community extended

- exit-address-family

- !

- address-family ipv4 vrf Emma

- neighbor 172.16.50.8 remote-as 500

- neighbor 172.16.50.8 activate

- exit-address-family

- !

- address-family ipv4 vrf Gvtel

- neighbor 172.16.20.5 remote-as 200

- neighbor 172.16.20.5 activate

- exit-address-family

- !

- address-family ipv4 vrf Venus

- neighbor 172.16.10.4 remote-as 100

- neighbor 172.16.10.4 activate

- exit-address-family

- !

- !

- end

- PE1#

- PE2#sh run

- Building configuration...

- !

- ! Last configuration change at 02:15:38 UTC Wed Mar 16 2022

- !

- hostname PE2

- !

- ip vrf Emma

- rd 525:3

- route-target export 525:3

- route-target import 525:3

- !

- ip vrf Gvtel

- rd 525:2

- route-target export 525:2

- route-target import 525:2

- !

- ip vrf Venus

- rd 525:1

- route-target export 525:1

- route-target import 525:1

- !

- !

- interface Loopback0

- ip address 3.3.3.3 255.255.255.255

- !

- interface GigabitEthernet0/0

- ip vrf forwarding Venus

- ip address 172.16.30.3 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- interface GigabitEthernet0/1

- ip address 100.100.100.3 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- interface GigabitEthernet0/2

- ip vrf forwarding Gvtel

- ip address 172.16.40.3 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- interface GigabitEthernet0/3

- ip vrf forwarding Emma

- ip address 172.16.60.3 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- mpls ip

- !

- router ospf 1

- network 3.3.3.3 0.0.0.0 area 0

- network 100.100.100.3 0.0.0.0 area 0

- !

- router bgp 525

- bgp log-neighbor-changes

- neighbor 1.1.1.1 remote-as 525

- neighbor 1.1.1.1 update-source Loopback0

- neighbor 172.16.30.6 remote-as 300

- neighbor 172.16.40.7 remote-as 400

- neighbor 172.16.60.9 remote-as 600

- !

- address-family vpnv4

- neighbor 1.1.1.1 activate

- neighbor 1.1.1.1 send-community extended

- exit-address-family

- !

- address-family ipv4 vrf Emma

- neighbor 172.16.60.9 remote-as 600

- neighbor 172.16.60.9 activate

- exit-address-family

- !

- address-family ipv4 vrf Gvtel

- neighbor 172.16.40.7 remote-as 400

- neighbor 172.16.40.7 activate

- exit-address-family

- !

- address-family ipv4 vrf Venus

- neighbor 172.16.30.6 remote-as 300

- neighbor 172.16.30.6 activate

- exit-address-family

- !

- !

- end

- PE2#

- CE1V#sh run

- Building configuration...

- !

- ! Last configuration change at 05:43:46 UTC Wed Mar 16 2022

- !

- hostname CE1V

- !

- !

- interface GigabitEthernet0/0

- ip address 192.168.10.4 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- interface GigabitEthernet0/1

- ip address 172.16.10.4 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- router bgp 100

- bgp log-neighbor-changes

- network 192.168.10.0

- neighbor 172.16.10.1 remote-as 525

- !

- end

- CE1V#

- CE2V#sh run

- Building configuration...

- !

- ! Last configuration change at 05:47:04 UTC Wed Mar 16 2022

- !

- hostname CE2V

- !

- interface GigabitEthernet0/0

- ip address 172.16.30.6 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- interface GigabitEthernet0/1

- ip address 192.168.20.6 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- router bgp 300

- bgp log-neighbor-changes

- network 192.168.20.0

- neighbor 172.16.30.3 remote-as 525

- !

- !

- end

- CE2V#

- CE1E#sh run

- Building configuration...

- !

- !

- hostname CE1E

- !

- interface GigabitEthernet0/0

- ip address 172.16.50.8 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- interface GigabitEthernet0/1

- ip address 192.168.10.8 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- router bgp 500

- bgp log-neighbor-changes

- network 192.168.10.0

- neighbor 172.16.50.1 remote-as 525

- !

- !

- end

- CE1E#

- CE2E#sh run

- Building configuration...

- !

- !

- hostname CE2E

- !

- !

- interface GigabitEthernet0/0

- ip address 172.16.60.9 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- interface GigabitEthernet0/1

- ip address 192.168.20.9 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- router bgp 600

- bgp log-neighbor-changes

- network 192.168.20.0

- neighbor 172.16.60.3 remote-as 525

- !

- !

- end

- CE2E#

- CE1G#sh run

- Building configuration...

- !

- hostname CE1G

- !

- !

- interface GigabitEthernet0/0

- ip address 192.168.10.5 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- interface GigabitEthernet0/2

- ip address 172.16.20.5 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- router bgp 200

- bgp log-neighbor-changes

- network 192.168.10.0

- neighbor 172.16.20.1 remote-as 525

- !

- !

- end

- CE1G#

- CE2G#sh run

- Building configuration...

- !

- hostname CE2G

- !

- !

- interface GigabitEthernet0/0

- ip address 192.168.20.7 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- interface GigabitEthernet0/2

- ip address 172.16.40.7 255.255.255.0

- duplex auto

- speed auto

- media-type rj45

- !

- !

- router bgp 400

- bgp log-neighbor-changes

- network 192.168.20.0

- neighbor 172.16.40.3 remote-as 525

- !

- !

- end

- CE2G#

- VPC11> ip 192.168.10.10/24 gateway 192.168.10.4

- VPC9> ip 192.168.20.10/24 gateway 192.168.20.6

- VPC14> ip 192.168.10.11/24 gateway 192.168.10.5

- VPC15> ip 192.168.20.11/24 gateway 192.168.20.7

- VPC10> ip 192.168.10.20/24 gateway 192.168.10.8

- VPC8> ip 192.168.20.20/24 gateway 192.168.20.9

- Quelques tests en images

- Ping entre le site Venus(AS100) et Venus(AS300)

- Ping entre le site Emma(AS500) et Emma(AS600)

- Ping entre le site Gvtel(AS200) et Gvtel(AS400)